WVU Defend the Republic

Code Documentation

Spring 2024

# Index

## Attacker Blimp

Overview

Nodes

Detect_cpp

Balloon_server

goal_server

Esc_driver

F_to_esc

Game_controller_node

Inv_kine

Joy_to_esc

Mode_switch

Pi_controller

Read_altitude

Read_imu

## Defender Blimp (Bomber)

Overview

Nodes

Baro_cntrl

Bomber_cntrl

Detect_cpp

Esc_driver

random_walk

Read_altitude

# Attacker Blimp

# Overview

# Nodes

## detect_client

| detect_cpp |  |
| --- | --- |
| Location | wvu_blimps_ros2_src/sensors_cpp/src/detect_client.cpp |
| File Type | C++ |
| Executable Name | detect_cpp |
| Dependencies |  |
| rclcpp | ROS2 cpp communication package |
| camera_coord | custom interface from blimp_interfaces to publish the camera coordinates |
| joy | interfaces from sensor_msgs/msg to subscribe to the /joy topic and read controller inputs. |
| OpenCV | library for computer vision to for image detection |
| vector | vector object used for sending camera data |
| Parameters | None |
| Subscriptions | /joy |
| Publications | /cam_data |

### Description

This node acts as the client node between the balloon_server and the goal_server. Every time this node begins to spin, it will open and begin running the camera. Then, the code will reach two flags that will control what the node will then do with the camera running. the  two flags that determine this are the control_mode flag and the cam_mode flag.

The control_mode flag determines whether the blimp is in manual mode or autonomous mode. This can be controlled by the pilot by pressing X on the Xbox controller to switch between modes. When the flag is true, the blimp is in manual mode, and when it’s false, it's in autonomous mode. In manual mode, the frame will not be captured and nothing will be published. In autonomous mode, the frame will be captured into an OpenCV matrix. Then it is converted into a vector to be sent over the service to the proper server nodes.

The cam_mode flag determines which server the frame vector should be sent to. When the flag is true, it is sent to the balloon_server node to complete the balloon_detection. When the flag is false, it is sent to the goal_server node to complete the goal detection.

After the server completes its computation and sends the data back, that contains a boolean hasDetection that determines if the server’s algorithm has a detection, and two integers, x_coord and y_coord,  that contain the X and Y coordinates for the center of the detected object. If the hasDetection is true, it will then publish the X and Y coordinates.

## balloon_server

| balloon_server |  |
| --- | --- |
| Location | wvu_blimps_ros2_src/sensors_cpp/src/balloon_server.cpp |
| File Type | C++ |
| Executable Name | balloon_server |
| Dependencies |  |
| rclcpp | ROS2 cpp communication package |
| OpenCV | library for computer vision to for image detection |
| vector | vector object used for sending camera data |
| Parameters | None |
| Subscriptions | None? |
| Publications | None? |

### Description

This is a server node used to compute the balloon detection algorithms using the frame sent by the detect_cpp node. This server will use the defined range of  HSV values and look for the area with the highest density of those values. The program will then draw a circle around the detected density. If the circle is bigger than the minimum radius of the center coordinates of the circle, it will then prep the data to be sent back through the service to the detect_cpp node. It will set the hasDetection boolean set to true, along with the integers x and y to the detected X and Y coordinates. If the algorithm does not generate a detection, it will then set hasDetection to false and then set the X and Y coordinates to 0.

## goal_server

| goal_server |  |
| --- | --- |
| Location | wvu_blimps_ros2_src/sensors_cpp/src/goal_server.cpp |
| File Type | C++ |
| Executable Name | goal_server |
| Dependencies |  |
| rclcpp | ROS2 cpp communication package |
| OpenCV | library for computer vision to for image detection |
| vector | vector object used for sending camera data |
| Parameters | None |
| Subscriptions | None? |
| Publications | None? |

### Description

This is a server node used to compute the goal detection algorithms using the frame sent by the detect_cpp node. We define a set range of HSV values to focus on the colors we need. From there, we find the edges of the color detected using cv2.canny. Using those edges, the openCV method, HoughLinesP, finds probabilistic lines and saves their x1, y1, x2, y2. Using these 2 points, we find the midpoints of the line. Taking the max and min X of the midpoints (meaning the 2 vertical sides of the goal) and the max and min Y (meaning the 2 horizontal sides of the goal) we find the average of those to give us the center of the detected goal. If the algorithm does detect a center of the goal, it will then set the hasDetection boolean set to true, along with the integers x and y to the detected X and Y coordinates. If the algorithm does not generate a detection, it will then set hasDetection to false and then set the X and Y coordinates to 0.

## esc_driver

| esc_driver |  |
| --- | --- |
| Location | wvu_blimps_ros2_src/controls/controls/esc_driver.py |
| File Type | python3 |
| Executable Name | esc_driver |
| Dependencies |  |
| rclpy | ROS2 python communications package |
| EscInput | custom interface from blimp_interfaces/msg for publishing and reading pwm inputs. |
| Joy | interfaces from sensor_msgs/msg to subscribe to the /joy topic and read controller inputs. |
| pigpio | library used to change the pwm signals on the specified pins. |
| os | allows the program to run an command in the terminal |
| time | used for sleep or getting the local time |
| subprocess | used to access the bluetooth stuff |
| Parameters | MAC: import the MAC address of the current controller. Default is 68:6C:E6:73:04:62 |
| Subscriptions | /joy |
| Publications | None |

### Description

This program is used to take in an ESC input and apply them to the motors using pigpio. The ESC inputs come from subscribing to the mode switcher.

## F_to_esc

| F_to_esc |  |
| --- | --- |
| Location | wvu_blimps_ros2_src/sensors_cpp/src/force_to_ESC_input.cpp |
| File Type | C++ |
| Executable Name | F_to_esc |
| Dependencies |  |
| rclcpp | ROS2 cpp communication package |
| card_coord | custom interface from blimp_interfaces/msg for publishing and reading cartesian. |
| esc_input | custom interface from blimp_interfaces/msg for publishing and reading pwm inputs. |
| Eigen/Dense | for doing matrix math in C++ |
| cmath | for more math functionality in C++ |
| Parameters | None |
| Subscriptions | /force |
| Publications | /ESC_balloon_input |

### Description

This program subscribes to the forces importing from the inv_kine nodes and uses a pseudo inverse method to convert the forces from the blimps body frame to motor inputs.

## game_controller_node

| game_controller_node |  |
| --- | --- |
| Subscriptions | /joyfeedback |
| Publications | /joy |

### Description

This node is from the Joy package install. It is used to read the inputs from the xbox controller and publishes to the /joy topic. For more information please visit the documentation websites. https://index.ros.org/p/joy/

## inv_kine

| inv_kine |  |  |
| --- | --- | --- |
| Location | wvu_blimps_ros2_src/sensors_cpp/src/inv_kine.cpp |  |
| File Type | C++ |  |
| Executable Name | dynamic_model |  |
| Dependencies |  |  |
| rclcpp | ROS2 cpp communication package |  |
| imu_data | custom package |  |
| card_coord |  |  |
| baro_data |  |  |
| esc_input | custom interface from blimp_interfaces/msg for publishing and reading pwm inputs. |  |
| Eigen/Dense | for doing matrix math in C++ |  |
| cmath | for more math functionality in C++ |  |
| Parameters | Rho_air (default=1.225) | Air density is used to find buoyancy of a balloon. |
|  | Buoyancy (default = Weight of Blimp) |  |
| Subscriptions | /imu_data |  |
|  | /barometer_data |  |
|  | /balloon_input |  |
| Publications | /forces |  |

### Description

Uses Information from IMU, barometer, and balloon input to model vehicles state. IMU data includes orientation, angular velocity, linear acceleration. Barometer data is used to approximate vertical velocity. Balloon input is used to determine vertical acceleration and yaw acceleration.  This information is run through the dynamic model and forces and moments are published.

## Joy_to_esc

| joy_to_esc |  |
| --- | --- |
| Location | wvu_blimps_ros2_src/controls/controls/joy_to_esc_input.py |
| File Type | python3 |
| Executable Name | joy_to_esc |
| Dependencies |  |
| rclpy | ROS2 python communications package |
| EscInput | custom interface from blimp_interfaces/msg for publishing and reading pwm inputs. |
| Joy | interfaces from sensor_msgs/msg to subscribe to the /joy topic and read controller inputs. |
| time | used for sleep or getting the local time |
| Parameters |  |
| Subscriptions |  |
| Publications |  |

### Description

This program is used to take in an ESC input and apply them to the motors using pigpio. The ESC inputs come from subscribing to the mode switcher.

## mode_switch

| mode_switch |  |
| --- | --- |
| Location | wvu_blimps_ros2_src/controls/controls |
| File Type | python3 |
| Executable Name | mode_switch |
| Dependencies |  |
| EscInput | custom interface from blimp_interfaces/msg for publishing and reading pwm inputs. |
| CameraCoord |  |
| Joy | interfaces from sensor_msgs/msg to subscribe to the /joy topic and read controller inputs. |
| time | used for sleep or getting the local time |
| Parameters | None |
| Subscriptions | /ESC_Manual_input |
|  | /ESC_balloon_input |
|  | /joy |
| Publications | /ESC_input |

### Description

## pi_controller

| pi_controller |  |  |
| --- | --- | --- |
| Location | wvu_blimps_ros2_src/sensors_cpp/src/test.cpp |  |
| File Type | C++ |  |
| Executable Name | pi_controller |  |
| Dependencies |  |  |
| rclpy | ROS2 python communications package |  |
| cart_coord |  |  |
| camera_coord | custom interface from blimp_interfaces to publish the camera coordinates |  |
| baro_data |  |  |
| vector | vector object used for sending camera data |  |
| cmath | for more math functionality in C++ |  |
| Parameters | iheight | initial height of barometer when program starts |
|  | kpx | proportional gain for yaw acceleration |
|  | kix | integral gain for yaw acceleration |
|  | kpyu | proportional gain for vertical acceleration (in the upward direction) |
|  | kpyd | proportional gain for vertical acceleration (in the downward direction) |
|  | kiy | integral gain for vertical acceleration |
|  | x_goal | goal coordinates of camera center, x_center of camera frame (320) |
|  | y_goal | goal coordinates of camera center, y_center of camera frame (240) |
|  | kpb | proportional gain for barometer |
| Subscriptions | /cam_data |  |
|  | /barometer_data |  |
| Publications | /balloon_input |  |

### Description

Uses Raspberry Pi camera v2.1 to approximate the center of a target via Hue Saturation Values (HSVs). Based on the x and y directional error between the target and the camera center (goal position). Vertical and yaw accelerations are tuned and sent to the inverse kinematic model.

## real_altitude

| read_altitude |  |  |
| --- | --- | --- |
| Location | wvu_blimps_ros2_src/sensors/sensors/barometer.py |  |
| File Type | python3 |  |
| Executable Name | read_altitude |  |
| Dependencies |  |  |
| time | Used for sleep of getting local time |  |
| board |  |  |
| adafruit_bmp3xx |  |  |
| BaroData |  |  |
| Parameters | sea_level_pressure | pressure at sea level on a given data |
| Subscriptions | None |  |
| Publications | barometer_data |  |

### Description

Approximates the current altitude of the blimp in meters based on atmospheric conditions of the day. Barometer used on current system is an Adafruit BMP390.

## read_imu

| read_imu |  |
| --- | --- |
| Location | wvu_blimps_ros2_src/sensors_cpp/src/inv_kine.cpp |
| File Type | python3 |
| Executable Name | read_imu |
| Dependencies |  |
| rclpy | ROS2 python communication package |
| time | used for sleep or getting the local time |
| board |  |
| busio |  |
| adafruit_bno055 |  |
| numpy |  |
| math |  |
| ImuData |  |
| Parameters | None |
| Subscriptions | None |
| Publications | /imu_data |

### Description

Uses an adafruit BNO055 absolute orientation sensor to record euler angles, linear acceleration, and angular velocity that is used in the dynamic model for an estimation of vehicle state.

# Defender Blimp (Bomber)

# Overview

# Nodes

## baro_cntrl

| detect_cpp |  |
| --- | --- |
| Location |  |
| File Type |  |
| Executable Name |  |
| Dependencies |  |
| Parameters |  |
| Subscriptions |  |
| Publications |  |

### Description

## bomber_cntrl

| bomber_cntrl |  |
| --- | --- |
| Location | wvu_blimps_ros2_src/controls/controls/Bomber_Cntrl.py |
| File Type | python3 |
| Executable Name | bomber_cntrl |
| Dependencies |  |
| rclpy | RPS2 python communications package |
| EscInput |  |
| CameraCoord |  |
| Scipy |  |
| Numpy |  |
| Parameters | None |
| Subscriptions | /cam_data |
| Publications | /ESC_Bomber_input |

### Description

Uses the cam_data topic data to calculate the motor inputs to move the detected object into the center of the camera then publishes the inputs to ESC_Bomber_input

## detect_cpp

| detect_cpp |  |
| --- | --- |
| Location | wvu_blimps_ros2_src/sensors_cpp/src/detect_node.cpp |
| File Type | C++ |
| Executable Name | detect_cpp |
| Dependencies |  |
| rclcpp | ROS2 cpp communication package |
| camera_coord | custom interface from blimp_interfaces to publish the camera coordinates |
| joy | interfaces from sensor_msgs/msg to subscribe to the /joy topic and read controller inputs. |
| OpenCV | library for computer vision to for image detection |
| vector | vector object used for sending camera data |
| Parameters | None |
| Subscriptions | /joy |
| Publications | /cam_data |

### Description

This node captures a image from the camera everytime it runs. It has two modes detect balloon or detect goal. To change between these either of these modes the pilot needs to press the X button on the Xbox controller. In balloon detection mode, the program will use the defined range of  HSV values and look for the area with the highest density of those values. The program will then draw a circle around the detected density. If the circle is bigger than the minimum radius the center coordinates of the circle is sent to to an averaging function. This function will publish the center coordinates every amount of detections we define.

In goal detection, the program works similarly to balloon detection. We define a set range of HSV values to focus on the colors we need. From there, we find the edges of the color detected using cv2.canny. Using those edges, the openCV method, HoughLinesP, finds probabilistic lines and saves their x1, y1, x2, y2. Using these 2 points, we find the midpoints of the line. Taking the max and min X of the midpoints(meaning the 2 vertical sides of the goal) and the max and min Y(meaning the 2 horizontal sides of the goal) we find the average of those to give us the center of the detected goal.

## esc_driver

| esc_driver |  |
| --- | --- |
| Location | wvu_blimps_ros2_src/controls/controls/esc_driver.py |
| File Type | python3 |
| Executable Name | esc_driver |
| Dependencies |  |
| rclpy | ROS2 python communications package |
| EscInput | custom interface from blimp_interfaces/msg for publishing and reading pwm inputs. |
| Joy | interfaces from sensor_msgs/msg to subscribe to the /joy topic and read controller inputs. |
| pigpio | library used to change the pwm signals on the specified pins. |
| os | allows the program to run an command in the terminal |
| time | used for sleep or getting the local time |
| subprocess | used to access the bluetooth stuff |
| Parameters | MAC: import the MAC address of the current controller. Default is 68:6C:E6:73:04:62 |
| Subscriptions | /joy |

### Description

This program is used to take in an ESC input and apply them to the motors using pigpio. The ESC inputs come from subscribing to the mode switcher.

## random_walk

| random_walk |  |
| --- | --- |
| Location | wvu_blimps_ros2_src/controls/controls/random_walk.py |
| File Type | python3 |
| Executable Name | random_walk |
| Dependencies |  |
| rclpy | ROS2 python communication package |
| EscInput |  |
| random | package to generate pseudo random numbers |
| Parameters | None |
| Subscriptions | None |
| Publications | /ESC_random_input |

### Description

uses the random library to generate a random input for a motor and then publishes it to ESC_random_input that lasts for 2 seconds then it generates a random input.

## bomber_mux

| bomber_mux |  |
| --- | --- |
| Location | wvu_blimps_ros2_src/controls/controls/bomber_mux.py |
| File Type | python3 |
| Executable Name | mux |
| Dependencies |  |
| rclpy | ROS2 python communication package |
| EscInput |  |
| time | used for sleep or getting the local time |
| Parameters | None |
| Subscriptions | /ESC_Bomber_input |
|  | /ESC_random_input |
|  | /ESC_baro_input |
| Publications | /ESC_input |

### Description

It subscribes to the ESC_Bomber_input, ESC_random_input, ESC_Baro_input sets the height control based off of baro_input then decides whether to use the random walk motor inputs or the bomber input based on the time from the last detection. If it is in bomber mode it will wait 10 seconds from the last inputs then if it hasn't changed by then it switches to random walk. If it is in random walk it will switch back to the bomber control if the bomber inputs change.

## read_altitude

| read_altitude |  |  |
| --- | --- | --- |
| Location | wvu_blimps_ros2_src/sensors/sensors/barometer.py |  |
| File Type | python3 |  |
| Executable Name | read_altitude |  |
| Dependencies |  |  |
| time | Used for sleep or getting the local time |  |
| board |  |  |
| adafriut_bmp3xx |  |  |
| BaroData |  |  |
| Parameters | sea_level_pressure | pressure at sea level on a given day |
| Subscriptions | None |  |
| Publications | /barometer_data |  |

### Description

Approximates the current altitude of the blimp in meters based on atmospheric conditions of the day. Barometer used on current system is an Adafruit BMP390