# Xbox Controller Mac Addresses:

White Controller: 68:6C:E6:73:04:62

red controller: A8:8C:3E:59:B5:C4

Connecting bluetooth on Ubuntu follow these instructions: https://pimylifeup.com/xbox-controllers-raspberry-pi/

# Interface Issues:

sometimes blimp interfaces does not build run the following command and try again. its magic:

export MAKEFLAGS="-j1 -l1"

Github Pull Issues:

Sometimes when you try to pull files from a branch it says already up to date even though some files are not updated you need to remove the pycache with the command: rm -rf /launch/__pycache/(name of launch file)

# Linux, ROS2, and other functionalities cheat sheet:

## Intro:

Important commands to remember. Feel free to add commands to this document that you might think will be helpful in the future to not forget. Please try your best to add a description of each command as well for others to know what they do.

## Useful Commands:

| Command | Description |
| --- | --- |
| colcon build | Compiles and builds every package in the ros2 workspace. Make sure you run the command when you are in just the workspace directory. Otherwise thing might get a little jank |
| colcon build --packages-select <ros2_pkg> | Compiles and builds just the specified package. Must be ran when in the ros2 workspace. Will allow for easier compiling because every package in the workspace wont be build. Change <ros2_package> to just your package name without the <> |
| source ~/.bashrc | re source the bashrc to make sure the new any new addition is sourced |
| ros2 run <ros2_pkg> <ros2_node_exc> | Runs the node <ros2_node> in the <ros2_pkg> package |
| ros2 node list | List all the running ros2 nodes |
| ros2 node info /node_name | Will give info on the node that is currently running and was specified |
| ros2 pkg create <my_python_pkg> --build-type ament_python --dependencies rclpy | Creates a new pythong package in the ros2 workspace |
| ros2 run <ros2_pkg> <ros2_node_exc> --ros-args -r __node:=abc | It is not kosher to run two ros2 nodes of the same name at the same time. This function will allow us to run the same node just under a different name. <ros2_pkg> is the ros package that has the node we want to run. <ros2_node_exc> is the node executable that we want to run. abc is the new name of the node we want to specify. |
| colcon build --packages-select <ros2_pkg> --symlink-install | If a file in the pkg is a executable this will allow you to never have to compile it after changing the contents. |
| ros2 topic list | lists all the currently running topics |
| ros2 topic echo <topic name> | echoes the topic output |

# Libraries installs needed for Drone:

- ROS2

  - Follow ros2 setup instructions (Ubuntu (Debian packages) — ROS 2 Documentation: Humble documentation)

- OpenCV (DO EACH OF THESE IN ORDER (a through i !!!)

  - sudo apt install libraspberrypi-bin v4l-utils ros-humble-v4l2-camera

  - sudo apt install ros-humble-image-transport-plugins

  - type the following command: groups. check if there is a video option there

    - if video is not there run:sudo usermod -aG video <device_username>

  - restart the raspberry

  - sudo apt-get install raspi-config

  - sudo raspi-config

    - select interface>legacy camera>enable

    - select interface>SPI>enable

    - select Interface>I2C>enable

    - finish

  - reboot

  - vcgencmd get_camera

    - should show 1 is supported and 1 is detected

    - if camera is not connected it will say 1 is supported

- Colcon

  - sudo apt install python3-colcon-common-extensions

- pip3 installer

  - sudo apt install python3-pip

- Circuit python dependencies

  - pip3 install Adafruit-circuitpython-lis3dh

  - sudo pip3 install adafruit-circuitpython-bno055

  - sudo pip3 install adafruit-circuitpython-bmp3xx

  - sudo pip3 install adafruit-circuitpython-bno08x

- Setup for the BNO085.

  - sudo nano i2c1-frequency-overlay.dts

  - copy and paste this in the file:

// i2c1-frequency-overlay.dts

/dts-v1/;

/plugin/;

/ {

compatible = "brcm,bcm2711";

fragment@0 {

target = <&i2c1>;

__overlay__ {

clock-frequency = <800000>; // Set your desired frequency here (e.g., 800 kHz)

};

};

};

  - dtc -I dts -O dtb -o i2c1-frequency-overlay.dtbo i2c1-frequency-overlay.dts

  - sudo cp i2c1-frequency-overlay.dtbo /boot/firmware/overlays/

  - sudo nano /boot/firmware/config.txt

  - add the following to the bottom of the config.txt

# Added below to change i2c frequency from 100kHz [default] to 800kHz [fast mode]

dtparam=i2c_arm_baudrate=800000

dtoverlay=i2c1-frequency-overlay

  - sudo reboot

  - check the baudrate this will display the number we put in, should be 8000000:

echo 0x$(xxd /sys/class/i2c-adapter/i2c-1/of_node/clock-frequency | cut -f 2,3 -d ' ' --output-delimiter='') | xargs printf "%d\n"

- RPi.GPIO

  - pip3 install RPi.GPIO

- joy ros2 package

  - sudo apt install ros-humble-joy

- pigpio

  - wget https://github.com/joan2937/pigpio/archive/master.zip

  - sudo apt install unzip

  - unzip master.zip

  - cd pigpio-master

  - make

  - sudo make install

- SCIPY

  - sudo pip3 install scipy

# Library Installs for C++ (not needed for current iteration):

- adafruit lis3dh

  - git clone https://github.com/adafruit/Adafruit_LIS3DH.git

  - cd Adafruit_LIS3DH

  - to/project (not done yet for amass no project made yet scotty)

- wiringpi (replacement for RPi.GPIO)

  - sudo apt-get install libwiringpi-dev (skipped cause we dont need scotty)

- pigpiod

  - wget https://github.com/joan2937/pigpio/archive/master.zip

  - unzip master.zip

  - cd pigpio-master

  - make

  - sudo make install

https://learn.adafruit.com/adafruit-bno055-absolute-orientation-sensor